Docu of Foraging_Task for Experimenters

# Study preparations (once a study starts)

- Go to “supplementaryFiles\ConditionFiles” and open the script “prepareConditionFiles.m”. At the top you can specify the number of condition files you want by giving start- and end- numbers. Below it are several global settings that apply to all participants – if you wish to change them.
Start the script to get your desired amount of condition files. For each participant will be created 2 files: cond_xxxxxx_1.mat for session 1 and cond_xxxxxx_2.mat for session 2.

- In the script Foraging_Task.m, set the preferences at the top of the script so they match your setup. Refer to section “Preferences” for more detailed explanation and further instructions.

# Preferences

doDebug – if set to 1, the task-window will be shown as smaller part of your screen, so you can access the Matlab Command Window. Also the cursor won’t be hidden.

doJoystick - if set to 1, the task will be controlled by turning a joystick to the left/right. You need to run the script “supplementaryFiles\Joystick\CalibrateJoystick.m”once for preparation.

doGFD – if set to 1, the task will be controlled by a 2-handed Gripforce-Device, X-device for left hand and Y-device for right hand. Run the script “supplementaryfiles\Gripforce\CalibrateGripforceDevice.m” once, in order to test the device.
Additionally, check at the end of function subjQuery() the dynamic path to the individual calibration files, that will be loaded (currently from the EAT-task).

In case “doJoystick = 0” and “doGFD = 0” the task is controlled by keys, which are noted in the command-window at startup.

doMRITrigger – if set to 1, the task waits for MRI trigger signals. Make sure that the function “waitForScannerTrigger()” has appropriate settings for your local MRI-scanner.

doTTL – if set to 1, the task will send TTL-triggers (to e.g. Biopac by using the TTL-trigger box from “The Black Box Toolkit”) make sure that the given com-port fits to the computer-settings – refer to the end of function “generalPreparations()” in the main script. The actual used trigger codes are set in “supplementaryFiles\ConditionFiles\prepareConditionFiles.m”.

doAttentionCheck – if set to 1, the task will conduct attention checks. A red asterisks will be shown in 25% of all trials above one target, to denote the mandatory choice of side. If participant fails 5 times to follow this, the reward will be halfed.

# Conduct a session

- It is highly recommended to switch-off all not necessary programms & switch-off network connection, because any other workload on the pc could worsen timing.

- If using a GripForce-Device, make sure to have a calibration file for this participant prepared. (refer to “Study preparations”)

- On starting the script

  - check the preference settings displayed in the command window

  - enter the Subject ID, (script will add padding zeros to match length 6)

  - enter the Session ID, (this will choose the respective condition file for the subject)

  - answer if it is a training run

    - if yes, answer if it is the long or the short version of the training.

    - if not, enter the “Condition ID” & the “Run ID”

  - enter the language

- the script/task can be aborted by pressing escape (doesn’t react instantly)

# Outputs

Find the output in folder .\data - after loading one of the *.mat files, the main-data (collected during the main trial-loop) can be found in “output.dataMat” - refer to .\Docs\Code Book.xlsx for a detailed explanation of the table.

The struct ”output.forceTraject” holds the force-feedback value of the GripForce-Device or Joystick,  with following rows: “trial-number; onset; mapped x-value; mapped y-value”.
Trial-number = 0 means feedback outside of the main trial-loop. In case a joystick was used, mapped x-value has a range from -100 to 100 (left to right), and there is no mapped y-value. In case a GripForce-Device was used, mapped x-value denotes the left and mapped y-value denotes the right side. Both in the range 0 to 100.

The struct “output.log” holds all data/events outside of the main trial-loop, like onsets of instructions slides or keypresses.